Lesson 2: Electrical Quantities and Multimeter Skills

Student practical and measurement record

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Measure DC voltage, resistance, and continuity safely. • Relate voltage, current, resistance, power, and capacity.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| voltage |  |
| current |  |
| resistance |  |
| power |  |
| capacity |  |
| DC |  |
| continuity |  |
| open circuit |  |
| short circuit |  |

Meter safety Black lead in COM. Red lead in V/Ω. Resistance and continuity are measured only with the circuit unpowered. Do not use the current socket in this lesson.

# 1. Before measuring

☐  I checked the black lead is in COM.

☐  I checked the red lead is in V/Ω.

☐  I selected DC voltage, resistance, or continuity as instructed.

☐  I confirmed whether the circuit should be powered or unpowered.

# 2. Measurement stations

| Station | Meter mode | Probe positions | Expected | Measured | Interpretation |
| --- | --- | --- | --- | --- | --- |
| A: cell/supply voltage | DC V |  |  |  |  |
| B: resistor | Ω |  |  |  |  |
| C: closed switch | Continuity |  |  |  |  |
| D: open switch | Continuity |  |  |  |  |
| E: buck input voltage | DC V |  |  |  |  |

# 3. Calculation

A pump is labelled 12V, 5W. Estimate its running current using I = P ÷ V.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

A battery is labelled 40Ah. Explain why this does not mean it forces 40A through the pump.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

# 4. Error analysis

| Mistake | What could happen? | Correct practice |
| --- | --- | --- |
| Measure resistance on a powered circuit |  |  |
| Red lead left in current socket |  |  |
| Probe tips bridge two nearby terminals |  |  |
| Voltage measured with both probes on the same wire |  |  |

# Exit ticket

Write one safe and one unsafe multimeter practice.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________