Lesson 3: Battery Safety, Switches, and Fuses

Protected power build and inspection record

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Explain how a fuse protects wiring. • Build the positive and negative distribution paths. • Pass an unpowered inspection.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| short circuit |  |
| overcurrent |  |
| fuse |  |
| series |  |
| polarity |  |
| distribution |  |
| insulation |  |

Non-negotiable rule The battery stays disconnected while wiring changes are made. Teacher approval is required before power is applied.

# 1. Reference netlist

BATTERY_POS -> MAIN_SWITCH.IN MAIN_SWITCH.OUT -> FUSE_3A.IN FUSE_3A.OUT -> POSITIVE_DISTRIBUTION BATTERY_NEG -> NEGATIVE_DISTRIBUTION

# 2. Build checklist

☐  Battery is disconnected.

☐  Positive and negative wires use clear color labels.

☐  Switch is in series with battery positive.

☐  Fuse is after the switch and before downstream distribution.

☐  Positive and negative are in separate WAGO/terminal blocks.

☐  No loose copper strands are visible.

☐  No conductor can touch the opposite polarity.

# 3. Inspection gate

| Check | Method | Result | Teacher initials |
| --- | --- | --- | --- |
| Polarity and labels | Visual |  |  |
| Fuse location | Netlist + visual |  |  |
| Switch operation | Continuity, unpowered |  |  |
| Positive-to-negative short | Continuity/resistance, unpowered |  |  |
| Wire retention | Gentle pull test |  |  |

# 4. Explain

Why must the fuse be close to the positive source rather than near the final component?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

What can a fuse protect against, and what can it not protect against?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

# Exit ticket

A student says, “It is only 12V, so it cannot be dangerous.” Write a technically accurate response.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________