Lesson 4: Step-Down Conversion with the LM2596

Setup, measurement, and sign-off record

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Identify converter input/output terminals. • Adjust and verify 5.00V output before connecting the ESP32.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| buck converter |  |
| input |  |
| output |  |
| step-down |  |
| regulated voltage |  |
| polarity |  |

Warning Raw battery voltage must never be connected to ESP32 5VIN. The ESP32 remains disconnected until the output is measured and signed off.

# 1. Label the converter

| Terminal | Connected to | Measured voltage relative to ground |
| --- | --- | --- |
| IN+ |  |  |
| IN− |  |  |
| OUT+ |  |  |
| OUT− |  |  |

# 2. Safe adjustment procedure

Power OFF and connect IN+ and IN− with correct polarity.

Keep the ESP32 disconnected.

Set meter to DC voltage; black probe on OUT− and red probe on OUT+.

Power ON and turn the adjustment screw slowly.

Set output to approximately 5.00V.

Power cycle and measure again.

Request teacher sign-off before connecting the ESP32.

# 3. Measurement record

| Condition | Input voltage | Output voltage | Pass? | Initials |
| --- | --- | --- | --- | --- |
| Before adjustment |  |  |  |  |
| After adjustment |  |  |  |  |
| After power cycle |  |  |  |  |
| Optional small-load test |  |  |  |  |

# 4. Reasoning

Why is measuring more reliable than trusting the previous adjustment or seller setting?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

What is the difference between OUT− and “no connection”?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

# Exit ticket

Write the safe power path from battery to ESP32 using arrows.

________________________________________________________________________________________________________________