Lesson 5: Powering the ESP32 Independently

External power and Serial Monitor test

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Connect verified 5V power to ESP32 5VIN/GND. • Distinguish power, programming, and serial communication.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| 5VIN |  |
| GND |  |
| USB CDC |  |
| port |  |
| baud rate |  |
| boot |  |
| serial |  |

Pre-power checkpoint Buck output has already been measured at 4.9–5.1V. Power is OFF while 5VIN and GND wires are installed.

# 1. Connections

LM2596.OUT+ -> ESP32.5VIN LM2596.OUT- -> ESP32.GND

☐  OUT+ wire is in 5VIN, not 3V3.

☐  OUT− wire is in GND.

☐  No raw battery wire reaches the ESP32.

☐  Connections are secure and polarity has been checked.

# 2. IDE settings

| Setting | Selected value |
| --- | --- |
| Board | ESP32S3 Dev Module |
| Port |  |
| USB CDC On Boot | Enabled if required |
| Serial Monitor baud | 115200 |
| Upload speed | 921600 or 115200 if troubleshooting |

# 3. Serial test sketch

void setup() { Serial.begin(115200); delay(2000); Serial.println("ESP32-S3 serial test OK"); } void loop() { Serial.println("hello"); delay(1000); }

# 4. Test evidence

| Test | Expected evidence | Observed |
| --- | --- | --- |
| Upload | IDE reports successful upload |  |
| Boot | Board powers from external 5V |  |
| Serial | Readable “hello” at 115200 |  |
| Reset | Output restarts after reset |  |

# 5. Explain the roles of USB

Power, programming/data transfer, and Serial Monitor are related but separate. Explain each.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________