Lesson 6: Digital Temperature Sensing with the DS18B20

Wiring, libraries, code, and response test

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Wire the DS18B20 to GPIO4. • Use OneWire and DallasTemperature. • Explain the pull-up resistor.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| digital sensor |  |
| OneWire |  |
| data line |  |
| pull-up resistor |  |
| defined state |  |
| library |  |

# 1. Wiring netlist

ESP32.3V3 -> DS18B20.VCC ESP32.GND -> DS18B20.GND DS18B20.DATA -> ESP32.GPIO4 6.8kΩ resistor between DS18B20.DATA and ESP32.3V3

Check wire colors Probe wire colors are not universal. Confirm the supplied sensor documentation or teacher-labelled cable before applying power.

# 2. Libraries

☐  Install OneWire by Paul Stoffregen.

☐  Install DallasTemperature by Miles Burton.

☐  Confirm both include statements compile.

# 3. Test code

#include <OneWire.h> #include <DallasTemperature.h> #define DS18B20_PIN 4 OneWire oneWire(DS18B20_PIN); DallasTemperature sensors(&oneWire); void setup() { Serial.begin(115200); delay(1500); sensors.begin(); Serial.print("Sensors found: "); Serial.println(sensors.getDeviceCount()); } void loop() { sensors.requestTemperatures(); float tempC = sensors.getTempCByIndex(0); if (tempC == DEVICE_DISCONNECTED_C) { Serial.println("ERROR: sensor disconnected"); } else { Serial.print("Temperature_C,"); Serial.println(tempC, 2); } delay(2000); }

# 4. Response test

| Condition | Time | Temperature (°C) | Expected direction | Notes |
| --- | --- | --- | --- | --- |
| Room-temperature water |  |  |  |  |
| Cooler water |  |  |  |  |
| Warmer water |  |  |  |  |
| Return to room-temperature |  |  |  |  |

# 5. Explain

Why does the DATA line need a resistor to 3.3V?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

What would an error value tell you to check first?