Lesson 7: Conductivity and Analog Input

EC wiring, library installation, and sample comparison

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Read raw ADC and voltage from the EC board. • Use the DFRobot_EC library with temperature compensation. • Interpret units cautiously.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| conductivity |  |
| ion |  |
| analog |  |
| ADC |  |
| millivolt |  |
| mS/cm |  |
| µS/cm |  |
| calibration |  |
| temperature compensation |  |

# 1. Wiring

ESP32.3V3 -> EC_BOARD.VCC ESP32.GND -> EC_BOARD.GND EC_BOARD.AO -> ESP32.GPIO1 EC_BOARD.T1 -> NOT CONNECTED

# 2. Install DFRobot_EC

Open https://github.com/DFRobot/DFRobot_EC

Choose Code → Download ZIP.

In Arduino IDE, choose Sketch → Include Library → Add .ZIP Library.

If installation fails, unzip and place a folder named DFRobot_EC in Documents/Arduino/libraries so DFRobot_EC.h is directly inside it.

# 3. Test code

#include <OneWire.h> #include <DallasTemperature.h> #include <DFRobot_EC.h> #include <EEPROM.h> #define DS18B20_PIN 4 #define EC_PIN 1 OneWire oneWire(DS18B20_PIN); DallasTemperature tempSensor(&oneWire); DFRobot_EC ec; void setup() { Serial.begin(115200); EEPROM.begin(32); tempSensor.begin(); ec.begin(); analogReadResolution(12); analogSetPinAttenuation(EC_PIN, ADC_11db); Serial.println("temp_C,ec_raw,ec_mV,ec_mS_cm,ec_uS_cm"); } void loop() { tempSensor.requestTemperatures(); float tempC = tempSensor.getTempCByIndex(0); int raw = analogRead(EC_PIN); float mV = raw * (3300.0 / 4095.0); float mS = ec.readEC(mV, tempC); ec.calibration(mV, tempC); Serial.print(tempC, 2); Serial.print(','); Serial.print(raw); Serial.print(','); Serial.print(mV, 1); Serial.print(','); Serial.print(mS, 3); Serial.print(','); Serial.println(mS * 1000.0, 1); delay(2000); }

# 4. Sample comparison

| Sample | Trial 1 mS/cm | Trial 2 | Trial 3 | Mean | Prediction supported? |
| --- | --- | --- | --- | --- | --- |
| Filtered water |  |  |  |  |  |
| Tap water |  |  |  |  |  |
| Weak salt solution |  |  |  |  |  |

# 5. Interpret the units

Convert 1.413 mS/cm to µS/cm: __________________________

Why should today’s EC values be described as estimated or relative?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

Probe care Rinse between samples. Keep immersion depth and waiting time consistent. Do not let salt solution dry on the probe.