Lesson 8: Turbidity and Voltage Dividers

Build, measure, and protect GPIO2

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Power the turbidity board from 5V. • Construct and verify a resistor divider. • Read the safe divided signal on GPIO2.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| turbidity |  |
| analog output |  |
| voltage divider |  |
| raw signal |  |
| measurement point |  |
| ground |  |
| ratio |  |

Do not skip the measurement gate GPIO2 stays disconnected until the divider point has been measured below 3.3V.

# 1. Wiring netlist

LM2596.OUT+ (5V) -> TURBIDITY.+ LM2596.OUT- -> TURBIDITY.- TURBIDITY switch -> A (analog) TURBIDITY.A/D -> 10kΩ -> DIVIDER_POINT DIVIDER_POINT -> 20kΩ -> GND DIVIDER_POINT -> ESP32.GPIO2 [ONLY AFTER VOLTAGE CHECK]

# 2. Resistor check

| Resistor | Color bands or measured resistance | Position |
| --- | --- | --- |
| 10kΩ |  | A/D to divider point |
| 20kΩ |  | Divider point to GND |

# 3. Multimeter gate

| Measurement | Black probe | Red probe | Expected | Measured | Pass? |
| --- | --- | --- | --- | --- | --- |
| Sensor power | GND | Turbidity + | About 5V |  |  |
| Raw signal | GND | A/D | 0–about 4.5V |  |  |
| Safe divider | GND | Divider point | Below 3.3V |  |  |

# 4. Calculation

The divider output fraction is 20k ÷ (10k + 20k) = ____________ of the raw signal.

Expected divider output when raw A/D = 4.5V: __________________________

# 5. Test code

#define TURBIDITY_PIN 2 const float DIVIDER_MULTIPLIER = 1.5; void setup() { Serial.begin(115200); analogReadResolution(12); analogSetPinAttenuation(TURBIDITY_PIN, ADC_11db); Serial.println("raw,gpio_V,estimated_sensor_V"); } void loop() { int raw = analogRead(TURBIDITY_PIN); float gpioV = raw * (3.3 / 4095.0); float sensorV = gpioV * DIVIDER_MULTIPLIER; Serial.print(raw); Serial.print(','); Serial.print(gpioV, 3); Serial.print(','); Serial.println(sensorV, 3); delay(1000); }

# 6. Explain the 20kΩ resistor

Why does the 20kΩ resistor connect from the GPIO2 point to ground? Why is a 10kΩ series resistor alone not the same thing?