Lesson 9: Integrating All Three Sensors

Wiring audit, combined code, and troubleshooting log

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Combine three sensors without losing safe power and signal conditions. • Use structured fault isolation.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| integration |  |
| pin definition |  |
| common ground |  |
| subsystem |  |
| fault isolation |  |
| hypothesis |  |
| evidence |  |

# 1. Pin and power map

| Device | Power | Ground | Signal pin | Extra component |
| --- | --- | --- | --- | --- |
| DS18B20 | 3.3V | GND | GPIO4 | 6.8k pull-up to 3.3V |
| EC board | 3.3V | GND | AO → GPIO1 | T1 unused |
| Turbidity board | 5V | GND | Divider point → GPIO2 | 10k/20k divider |

# 2. Integration checklist

☐  Power OFF while wiring.

☐  All grounds connect to one common-ground net.

☐  Each sensor has already worked by itself.

☐  GPIO1, GPIO2, and GPIO4 match the code.

☐  Turbidity divider point was measured below 3.3V.

☐  No raw 12V reaches ESP32 or sensors.

# 3. Integrated test code

#include <OneWire.h> #include <DallasTemperature.h> #include <DFRobot_EC.h> #include <EEPROM.h> #define DS18B20_PIN 4 #define EC_PIN 1 #define TURBIDITY_PIN 2 OneWire oneWire(DS18B20_PIN); DallasTemperature tempSensor(&oneWire); DFRobot_EC ec; int averageAnalog(int pin) { long total = 0; for (int i = 0; i < 20; i++) { total += analogRead(pin); delay(10); } return total / 20; } void setup() { Serial.begin(115200); EEPROM.begin(32); tempSensor.begin(); ec.begin(); analogReadResolution(12); analogSetPinAttenuation(EC_PIN, ADC_11db); analogSetPinAttenuation(TURBIDITY_PIN, ADC_11db); Serial.println("temp_C,ec_raw,ec_mV,ec_mS_cm,turb_raw,turb_gpio_V"); } void loop() { tempSensor.requestTemperatures(); float tempC = tempSensor.getTempCByIndex(0); int ecRaw = averageAnalog(EC_PIN); float ecMV = ecRaw * (3300.0 / 4095.0); float ecMS = ec.readEC(ecMV, tempC); ec.calibration(ecMV, tempC); int turbRaw = averageAnalog(TURBIDITY_PIN); float turbGPIOV = turbRaw * (3.3 / 4095.0); Serial.print(tempC, 2); Serial.print(','); Serial.print(ecRaw); Serial.print(','); Serial.print(ecMV, 1); Serial.print(','); Serial.print(ecMS, 3); Serial.print(','); Serial.print(turbRaw); Serial.print(','); Serial.println(turbGPIOV, 3); delay(2000); }

# 4. First integrated readings

| Reading | Temperature | EC raw | EC mS/cm | Turbidity raw | Turbidity GPIO V |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

# 5. Troubleshooting log

| Symptom | Evidence checked | Hypothesis | One change made | Result |
| --- | --- | --- | --- | --- |

Recommended order 1) power voltage, 2) common ground, 3) signal voltage, 4) physical pin, 5) code pin definition, 6) library/compile messages. Change one thing at a time.