Lesson 10: CSV Output and Timestamps

Structured logging and spreadsheet import

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Create elapsed timestamps with millis(). • Print consistent CSV rows every five seconds. • Import and graph the data.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| CSV |  |
| header |  |
| field |  |
| row |  |
| timestamp |  |
| millis() |  |
| interval |  |
| delimiter |  |

# 1. Diagnose the CSV

Valid example

time_ms,temp_C,ec_mS_cm,turbidity_raw 5000,23.81,0.412,2860 10000,23.81,0.417,2848 15000,23.88,0.415,2855

What would break the import? List three possible formatting mistakes.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

# 2. Logging code pattern

const unsigned long READ_INTERVAL_MS = 5000; unsigned long nextReading = 0; void loop() { unsigned long now = millis(); if (now >= nextReading) { nextReading = now + READ_INTERVAL_MS; // Read sensors here Serial.print(now); Serial.print(','); Serial.print(tempC, 2); Serial.print(','); Serial.print(ecMS, 3); Serial.print(','); Serial.println(turbRaw); } }

# 3. Header and fields

| Field name | Meaning | Unit | Example |
| --- | --- | --- | --- |
| time_ms | Elapsed time since boot | ms | 5000 |
| temp_C | Water temperature | °C | 23.81 |
| ec_mS_cm | Estimated conductivity | mS/cm | 0.412 |
| turbidity_raw | ESP32 ADC reading | ADC count | 2860 |

# 4. Data collection record

☐  Header prints once.

☐  Every row has the same number of commas.

☐  Readings are approximately five seconds apart.

☐  No commentary appears inside the CSV data.

☐  File is saved with .csv extension.

# 5. Spreadsheet task

Open the CSV in your spreadsheet program.

Verify each field appears in its own column.

Create one line graph against time.

Add axis labels, units, and a descriptive title.

Write a caption stating the sample and whether the values are calibrated.

# 6. Reflection

What does millis() tell us, and what does it not tell us? How could a future version record real date and time?