Lesson 13: Permanent Assembly and Strain Relief

Soldering, insulation, and harness inspection

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Make a clean electrical joint. • Insulate conductors and protect PCB pads from movement. • Build a traceable, serviceable harness.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| tinning |  |
| wetting |  |
| cold joint |  |
| solder bridge |  |
| strain relief |  |
| pigtail |  |
| ferrule |  |
| heat shrink |  |
| service loop |  |

Soldering safety Circuit unpowered. Safety glasses on. Iron returns to holder whenever it is not touching the work. Use ventilation. Wash hands after handling leaded solder.

# 1. Quality rubric

| Feature | Good | Needs correction |
| --- | --- | --- |
| Wire preparation | Correct strip length; strands twisted; light tinning | Loose strands; excessive stripped copper; large solder spike |
| Joint | Solder wets wire and pad; smooth fillet | Ball/blob, cold grainy surface, incomplete wetting |
| Bridges | No contact with adjacent pads | Solder bridge or loose strand |
| Insulation | Heat shrink covers exposed conductor | Bare resistor legs or solder exposed |
| Strain relief | Cable secured by insulation/clamp/tie | Force carried by pad or solder joint |
| Serviceability | Labels and connectors allow tracing/removal | Unlabelled permanent bundle |

# 2. Practice-joint sign-off

| Practice | Electrical continuity | No short | Mechanical inspection | Teacher initials |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |

# 3. Harness plan

| Connection | Wire color/label | Connector or solder method | Strain relief location |
| --- | --- | --- | --- |
| 5V |  |  |  |
| GND |  |  |  |
| GPIO1 EC |  |  |  |
| GPIO2 turbidity divider |  |  |  |
| GPIO4 temperature |  |  |  |

# 4. Final inspection

☐  Battery disconnected during assembly.

☐  No exposed positive conductor.

☐  No solder bridge between adjacent pads.

☐  Divider resistors and legs are insulated.

☐  Screw-terminal stranded wires use appropriate ferrules where required.

☐  Cable ties/clamps carry movement, not PCB pads.

☐  Every connection can be traced to the netlist.

☐  Continuity and no-short checks completed before power.

# 5. Reflection

Why is a solder joint not a strain-relief system? Describe how your build handles movement.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________