Lesson 14: Demonstration and Evaluation

Final test record, evidence, and reflection

Grade 9 MYP Design | From USB to Field System

| Name: __________________ | Class / team: __________________ | Date: __________________ |
| --- | --- | --- |

Today’s goals • Demonstrate the system safely. • Judge each criterion using evidence. • Propose technically justified improvements.

# Key vocabulary

| Term | Working definition |
| --- | --- |
| criterion |  |
| evidence |  |
| judgement |  |
| fault |  |
| limitation |  |
| enhancement |  |
| evaluation |  |

# 1. Pre-demonstration checklist

☐  Wiring has passed final inspection.

☐  Buck output has been measured.

☐  Turbidity divider point is below 3.3V.

☐  Code is saved and compiles.

☐  CSV header and rows are correct.

☐  Water sample and wet zone are ready.

☐  One team member can explain every major component.

# 2. Demonstration script

Show the protected power path and identify the fuse.

Show the buck-converter measurement.

Power the system and open Serial Monitor.

Explain GPIO4 temperature, GPIO1 conductivity, and GPIO2 turbidity.

Demonstrate a sensor response to a controlled sample change.

Capture CSV and open it in a spreadsheet.

State which readings are calibrated, estimated, or relative.

# 3. Criterion-based evaluation

| Success criterion | Evidence | Met / partly / not met | Improvement |
| --- | --- | --- | --- |
| Operates from external 12V power system |  |  |  |
| Fuse is correctly placed |  |  |  |
| Buck output is 4.9–5.1V |  |  |  |
| No raw 12V reaches ESP32/sensors |  |  |  |
| Temperature responds plausibly |  |  |  |
| EC responds to ion concentration |  |  |  |
| Turbidity responds to suspended material |  |  |  |
| GPIO2 remains below 3.3V |  |  |  |
| CSV rows occur at correct interval |  |  |  |
| Wiring has insulation and strain relief |  |  |  |
| Runs for at least ten minutes without reset |  |  |  |

# 4. Fault, limitation, or enhancement?

| Observation | Category | Why |
| --- | --- | --- |

# 5. Final evaluation

Most successful feature and evidence:

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

Most important limitation and evidence:

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

Highest-priority improvement, how it would work, and why it matters:

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

What did this unit teach you that a USB LED exercise did not?

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________